# Alternator Predictive Maintenance — Three-Iteration Summary

V10.6.2 (honest RUL baseline)  ·  V11 (lead-time heuristics)  ·  V11.1 (covariate RUL test)   |   25 trucks: 10 failed, 15 in service

# 1. The three questions, in plain language

For every truck we ask: WHICH truck is likely to fail? WHEN do alternators fail in general? And when will THIS specific truck fail?

WHICH — solved and frozen since V10.5.3: a risk score that separates failing from healthy trucks with 0.93/1.00 accuracy (AUROC). Identical in all three iterations.

WHEN (fleet) — known: alternators in this fleet wear out around 601 days of service (roughly 120,000 km). Identical in all three iterations.

WHEN (each truck) — the hard one. A dumb rule ('every truck fails at ~601 days, count down') predicts within ~50 days. Every sophisticated model has been off by ~140 days.

# 2. What each iteration did

V10.6.2 — proved the humbling baseline: the dumb fleet-clock rule beats the per-truck model (50 vs ~140 days error). With only 10 failures to learn from, individual failure dates were declared out of reach; the covariate idea was shelved untested.

V11 — hunted for earlier warning signs in the same six electrical signals. Found one good new signal: a failing truck's voltage recovers slowly after engine start. Raised warning coverage from 5 to 6 of the 10 failures with zero false alarms on healthy trucks, and made one warning 30 days earlier.

V11.1 — asked the obvious follow-up: if we feed those new warning signs into the failure-date math (like a doctor adding blood-test results to age), can we finally predict each truck's date? The answer is NO — and now it is proven, not assumed. Adding the signals made predictions slightly WORSE (149–162 vs 140 days error). One trap explained much of it: counting a truck's lifetime 'slow recovery' events mostly measures how long it has lived, not how sick it is. The question is now closed with strict no-peeking-into-the-future checks (all five honesty gates passed).

# 3. What you actually get (deployable today)

Risk ranking of in-service trucks (frozen classifier, AUROC 0.927).

Fleet replacement window: plan around ~601 days / ~120,000 km.

RED alarm — electrical fault-code storm (GED=2): fired for 2 of 10 failures (21-day and 1-day leads), never on a healthy truck.

YELLOW alarm (new in V11.1) — truck CURRENTLY showing the combined electrical-distress signature: 3 of 10 failures, never on a healthy truck. Feeds the maintenance recommendation directly.

A guardrail finding: always-on versions of the new alarms would cry wolf (up to 6 of 15 healthy trucks falsely flagged) — only the 'distressed right now' version is deployed.

# 4. Metrics — three versions side by side

Shared foundations (identical by design):

| Metric | V10.6.2 | V11 | V11.1 |
| --- | --- | --- | --- |
| Classifier AUROC (WHICH truck) | 0.9267 (frozen) | 0.9267 (frozen) | 0.9267 (frozen) |
| Fleet wear-out window | 601 d empirical | 601 d (by reference) | 601 d (by reference) |
| Weibull fit (shape / scale) | 5.17 / 771 d | — (not re-run) | 5.17 / 771 d (reproduced) |

Per-truck RUL accuracy (time-rewound LOVO, lower is better):

| Model | Day-MAE | PI coverage | Verdict |
| --- | --- | --- | --- |
| Fleet-clock dummy (601-day rule) | 49.7 d | — | still unbeaten |
| V10.6.2 model (no covariates) | 141.8 d | 0.867 | loses to dummy |
| V11.1 M0 (no covariates) | 140.4 d | 0.867 | ships (= V10.6.2) |
| V11.1 M1 (+ crank-recovery count) | 148.8 d | 0.833 | worse — exposure-confounded |
| V11.1 M2 (+ compound-vote state) | 162.2 d | 0.800 | worst |

Verdict: NO_IMPROVEMENT — covariates do not individualize RUL (third structural confirmation; gate-enforced honest fallback).

Warning / lead-time channels (failed-truck recall, healthy-truck false alarms):

| Channel | V10.6.2 | V11 | V11.1 |
| --- | --- | --- | --- |
| Strict-gate precursor recall | 5/10 | 6/10 (0/15 FA) | carried from V11 |
| GED=2 storm (RED alarm) | 2/10, 0/15 FA | 2/10, 0/15 FA | 2/10, 0/15 FA |
| Compound early-watch (YELLOW) | — | 4/10, 0/15 FA (window test) | 3/10, 0/15 FA (current-state, deployable) |
| Decision matrix | 2-D (risk × time) | — | 3-D (risk × time × emergency) |
| Best new signal | — | post-crank voltage recovery | same, as alarm input |

# 5. Bottom line

Same as before: which trucks are risky, and the ~601-day fleet replacement window.

Newly proven: individual failure dates are impossible with this data — stop chasing them.

Newly gained: a second zero-false-alarm warning light and a maintenance decision table that uses it.

## Appendix — number sources

| Figure | Source file |
| --- | --- |
| AUROC 0.9267 | V5.2_ALT/results/V10.5.3_20_5_ALT_final_report.json |
| 601 d window | V10.6.2_ALT/cache/rul/fleet_window.json |
| MAE 141.8 / dummy 49.7 | V10.6.2_ALT/cache/backtest/backtest_results.json |
| MAE 140.4 / 148.8 / 162.2 | V11.1_ALT/cache/backtest/backtest_results.json |
| Recall 5/10 vs 6/10, 0/15 FA | V11_ALT_heuristics results + cache/forensics |
| Early-watch 3/10, 0/15 FA | V11.1_ALT/cache/emergency/emergency_per_vin.csv |
| Five gates PASS | V11.1_ALT/results/V11.1_ALT_verification.json |